--------------------------------------------------------------------------------

ONLINE DOCUMENTATION

--------------------------------------------------------------------------------

Manufacturer Website: https://www.snapav.com

Support Site: https://www.snapav.com/shop/en/snapav/support

--------------------------------------------------------------------------------

CHANGELOG

--------------------------------------------------------------------------------

v1.1

Added digital coax output and analog output connections

v1.0

Initial Release

--------------------------------------------------------------------------------

OVERVIEW

--------------------------------------------------------------------------------

This driver integrates a SunBriteTV into your Control4 Project by use of IR transmittion. To

use this driver, please add the driver to the existing project and follow the setup

instructions below.

--------------------------------------------------------------------------------

SETUP

--------------------------------------------------------------------------------

1. Physically connect the IR cable from control system input to the television IR receiver.

2. In the connections tab, bind the IR input from the driver to the IR output

you're using for the connection.

i.e SUNBRITETV IR DRIVER INPUT ----- HC250 IR OUTPUT

--------------------------------------------------------------------------------

PROPERTIES

--------------------------------------------------------------------------------

SR250 RED BUTTON

Configurable button for navigation. Default is picture mode 1.

SR250 GREEN BUTTON

Configurable button for navigation. Default is picture mode 2.

SR250 YELLOW BUTTON

Configurable button for navigation. Default is picture mode 3.

SR250 BLUE BUTTON

Configurable button for navigation. Default is picture mode 4.

SR260 CUSTOM BUTTON ONE

Configurable button for navigation.

SR260 CUSTOM BUTTON TWO

Configurable button for navigation.

SR260 CUSTOM BUTTON THREE

Configurable button for navigation.

DEBUG MODE

Mode for debugging lua in the lua output window

--------------------------------------------------------------------------------

EVENTS

--------------------------------------------------------------------------------

POWER ON

POWER OFF

INPUT CHANGED

--------------------------------------------------------------------------------

VARIABLES

--------------------------------------------------------------------------------

CURRENT INPUT

POWER STATE

--------------------------------------------------------------------------------

ACTIONS

--------------------------------------------------------------------------------

PRINT DEVICE-SPECIFIC MAPPINGS

In the lua output, the device specific mappings will be printed in a list

PRINT BUTTON MAPPINGS

In the lua output, the proxy button mappings will be printed in a list

--------------------------------------------------------------------------------

DEVICE-SPECIFIC COMMANDS

--------------------------------------------------------------------------------

ASPECT - Press Button ASPECT

A D - Press Button Add/Delete

CANCEL - Press Button CANCEL

CC - Press Button Closed Captioned

CHANNEL DOWN - Press Button CHANNEL DOWN

CHANNEL LOCK - Press Button CHANNEL LOCK

CHANNEL UP - Press Button CHANNEL UP

DNR - Press Button DNR

DOWN - Press Button DOWN

ENTER - Press Button ENTER

FAVORITE A D - Press Button FAVORITE Add/Delete

FAVORITE DOWN - Press Button FAVORITE DOWN

FAVORITE UP - Press Button FAVORITE UP

FREEZE - Press Button FREEZE

GUIDE - Press Button GUIDE

INFO - Press Button INFO

INPUT AV - Press Button INPUT AV

INPUT COMPONENT 1 - Press Button INPUT COMPONENT 1

INPUT COMPONENT 2 - Press Button INPUT COMPONENT 2

INPUT HDBASE T - Press Button INPUT HDBASE T

INPUT HDMI 1 - Press Button INPUT HDMI 1

INPUT HDMI 2 - Press Button INPUT HDMI 2

INPUT NA - Press Button INPUT NA

INPUT PC VGA - Press Button INPUT PC VGA

INPUT TUNER - Press Button INPUT TUNER

INPUT USB MEDIA - Press Button INPUT USB MEDIA

LEFT - Press Button LEFT

MENU - Press Button MENU

MTS - Press Button MTS

MUTE TOGGLE - Press Button MUTE TOGGLE

NUMBER 0 - Press Button 0

NUMBER 1 - Press Button 1

NUMBER 2 - Press Button 2

NUMBER 3 - Press Button 3

NUMBER 4 - Press Button 4

NUMBER 5 - Press Button 5

NUMBER 6 - Press Button 6

NUMBER 7 - Press Button 7

NUMBER 8 - Press Button 8

NUMBER 9 - Press Button 9

OFF - Press Button OFF

ON - Press Button ON

PICTURE - Press Button PICTURE

PICTURE MODE 1 - Press Button PICTURE MODE 1

PICTURE MODE 2 - Press Button PICTURE MODE 2

PICTURE MODE 3 - Press Button PICTURE MODE 3

PICTURE MODE 4 - Press Button PICTURE MODE 4

PIP AUDIO - Press Button PIP AUDIO

PIP POSITION - Press Button PIP POSITION

PIP SIZE - Press Button PIP SIZE

PIP SOURCE - Press Button PIP SOURCE

PIP TOGGLE - Press Button PIP TOGGLE

PIP VIDEO - Press Button PIP VIDEO

RECALL - Press Button RECALL

RIGHT - Press Button RIGHT

SLEEP - Press Button SLEEP

SOUND - Press Button SOUND

SOURCE - Press Button SOURCE

SRS - Press Button SRS

UP - Press Button UP

VOLUME DOWN - Press Button VOL DOWN

VOLUME UP - Press Button VOL UP